ReadMe from: https://github.com/mtornga/PiSensorNetwork 

PiOT

Creating some IoT-type devices for around the house and having them report back to a main node. The current thrust of PiOT collects temperature and humidity data.

Result

Architecture:

Technologies Demonstrated:

- Docker

  - Creating containers in a network

  - Automating deployment of the stack

  - Exposing ports to localhost

- MySQL

  - Database and table creation

  - Light ETL

  - Event manager for scheduling actions

- Nifi

  - InvokeHTTP for scheduling HTTP GET requests

  - PutDatabaseRecord for converting JSON into INSERT statements

  - Processor templating

- Python3

  - Flask for hosting a RESTful API

  - Command-line arguments

  - Writing to a SQLite database

- SQLite

- Crontab

- DH22 Sensor

# Deploying on a new Pi

Deployed and tested on Raspberry Pi versions 3b+, 4, and Zero W with the full Raspian installation.

## Preparing the Raspian environment

Raspian was installed using the NOOBS thing where the OS is loaded on the SIM card for the user. I will try to script the installation of these other pre-reqs:

- Python3

- Adafruit DHT

- Git

sudo apt update sudo apt-get update && sudo apt-get upgrade -y (something like this) sudo apt-get install python3 sudo apt-get install git mkdir repos && cd repos git clone https://MarkTornga@bitbucket.org/MarkTornga/piot.git mkdir /home/pi/data then run CreateSensorDB

installing adafruit dht stuff sudo python3 -m pip install --upgrade pip setuptools wheel sudo pip3 install Adafruit_DHT git clone https://github.com/adafruit/Adafruit_Python_DHT.git sudo python3 setup.py install

Run pinout to get the GPIO info and then attach the sensor. Plus to power, minus to ground, out to a GPIO pin

Enable ssh access: sudo systemctl enable ssh sudo systemctl start ssh

Assign static IP

# Setting up the home base

In nifi, have to upload template, add template to tthe canvas, in DBCPConnectionPool controller service: Databse connection URL will need to do docker inspect network web to find the ip for the finaldb container have to set the password parameter